EXTRON   DVI DA DVI 分配放大器

Extron DVI DA2 和 DVI DA4 是带 2 路或 4 路输出的 DVI 分配放大器。两种型号均接受单链路 DVI 信号，并可将同样的信号分配到所连接的设备。它们提供到本地输出的 DDC（显示数据通道）路由，从而允许 EDID（扩展显示识别数据）的反馈，以确保视频的正常输出和资料的可靠显示。对于需要将高分辨率视频信号路由至多个目的地，并且从信号源到目的地之间为全数字信号路径的应用环境来说，这些 DVI 分配放大器无疑是理想之选。

主要特性

|  | 支持分辨率高达 1920x1200（包括 HDTV 1080p/60）的单链路 DVI-D 信号 |
| --- | --- |
|  | 自动输入电缆均衡 |
|  | DDC 路由至本地输出，用于 EDID 和 HDCP |
|  | 可机架安装的 1U 高、半机架宽的金属外壳 |

| 型号 | 产品说明 | 产品编号 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DVI DA2 | 带 2 路 DVI 输出 | 60-886-02 |  |  |  |  |  |
|  | DVI DA4 | 带 4 路 DVI 输出 | 60-922-01 |  |  |  |  |  |

Extron DVI DA2 和 DVI DA4 是带 2 路或 4 路输出的 DVI 分配放大器。两种型号均接受单链路 DVI 信号，并可将同样的信号分配到所连接的设备。它们提供到本地输出的 DDC（显示数据通道）路由，从而允许 EDID（扩展显示识别数据）的反馈，以确保视频的正常输出和资料的可靠显示。对于需要将高分辨率视频信号路由至多个目的地，并且从信号源到目的地之间为全数字信号路径的应用环境来说，这些 DVI 分配放大器无疑是理想之选。

| Video |  |
| --- | --- |
| Maximum bit rate | 4.95 Gbps (1.65 Gbps per color) |
| Maximum pixel clock | 165 MHz |
| Resolution range | Up to 1920x1200 or 1080p @ 60 Hz |
| Formats | RGB and YCbCr digital video |
| Standards | DVI 1.0, HDMI 1.2 |
| Video input |  |
| Number/signal type | 1 single link DVI-D (or HDMI*) |
| Connectors | 1 female DVI-I |
| Video output |  |
| Number/signal type |  |
| DVI DA2 | 2 single link DVI-D (or HDMI*): 1 main, 1 local |
| NOTE; Local output is used for DDC communications and hot plug detection.. |
| DVI DA4 | 4 single link DVI-D (or HDMI*) |
| Connectors | 2 or 4 female DVI-I (depending on model) |
| General |  |
| External power supply | 100 VAC to 240 VAC, 50-60 Hz, external; to 12 VDC, 1 A, regulated |
| Power input requirements | 12 VDC, 0.4 A |
| Temperature/humidity | Storage: -40 to +158 °F (-40 to +70 °C) / 10% to 90%, noncondensing Operating: +32 to +122 °F (0 to +50 °C) / 10% to 90%, noncondensing |
| Cooling | Convection, no vents |
| Mounting |  |
| Rack mount | Yes, with optional rack shelf kit |
| Furniture mount | Yes, with optional under-desk mounting kit |
| Enclosure type | Metal |
| Enclosure dimensions | 1.75" H x 8.75" W x 3.0" D (1U high, half rack wide) (4.4 cm H x 22.2 cm W x 7.6 cm D) (Depth excludes connectors.) |
| Product weight | 2.8 lbs (1.3 kg) |
| Shipping weight | 5 lbs (3 kg) |
| Vibration | ISTA 1A in carton (International Safe Transit Association) |
| Regulatory compliance |  |
| Safety | CE, CUL, UL |
| EMI/EMC | CE, C-tick, FCC Class A, ICES, VCCI |
| MTBF | 30,000 hours |
| Warranty | 3 years parts and labor |