EXTRON  ISS 506 无缝切换器

Extron ISS 506 是一款 6 路输入无缝切换器，可以接受多种视频信号，包括 RGB 计算机视频、HDTV 和标清视频。它将真正的无缝、无抖动切换与高级解析度转换技术相结合，可满足高品质、高分辨率视频演示的要求。ISS 506 具有各种演示增强特性，如独立的预览和节目输出、多种切换转换效果、徽标插入、字幕键入、内部测试图案和多种控制方法，为高端演示环境提供了多种高级功能。

主要特性

|  | 预览和节目输出功能 |
| --- | --- |
|  | 对复合视频、S-视频、分量视频、RGBHV 和 HDTV 信号进行解析度转换 |
|  | 多种转换效果，包括擦拭、溶解或快切 |
|  | 多徽标插入 |
|  | PIP – 画中画 |
|  | 字幕键入 |

| 型号 | 产品说明 | 产品编号 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ISS 506 | 标准型号 | 60-742-01 |  |  |  |  |  |
|  | ISS 506 SC | 带扫描转换输出 | 60-742-03 |  |  |  |  |  |
|  | ISS 506 DI/DVI | 带 SDI/HD-SDI 输入，DVI-D 输出 | 60-742-12 |  |  |  |  |  |
|  | ISS 506 DI/SC | 带 SDI/HD-SDI 输入，扫描转换输出 | 60-742-13 |  |  |  |  |  |
|  | ISS 506 DI/HD-SDI | 带 SDI/HD-SDI 输入和 HD-SDI 输出 | 60-742-14 |  |  |  |  |  |

Extron ISS 506 是一款 6 路输入无缝切换器，可以接受多种视频信号，包括 RGB 计算机视频、HDTV 和标清视频。它将真正的无缝、无抖动切换与高级解析度转换技术相结合，可满足高品质、高分辨率视频演示的要求。ISS 506 具有各种演示增强特性，如独立的预览和节目输出、多种切换转换效果、徽标插入、字幕键入、内部测试图案和多种控制方法，为高端演示环境提供了多种高级功能。

| Video input |  |
| --- | --- |
| Number/signal type | 4 RGBHV, RGBS, RGsB, RGBcvS, component video (interlaced or progressive), S-video, composite video 1 component video (interlaced), S-video, composite video 1 S-video, composite video, or optional digital component video (SDI or HDSDI) |
| Connectors | 4 x 5 female BNC 1 x 3 female BNC, 1 female BNC, 1 female 4-pin mini DIN 1 optional BNC for SDI/HDSDI |
| Nominal level | 1 Vp-p for Y of component video and S-video, and for composite video 0.7 Vp-p for RGB and for R-Y and B-Y of component video 0.3 Vp-p for C of S-video |
| Minimum/maximum levels | Analog: 0.0 V to 2.0 Vp-p with no offset at unity gain |
| Impedance | 75 ohms |
| Horizontal frequency | Autoscan 15 kHz to 100 kHz |
| Vertical frequency | Autoscan 50 Hz to 120 Hz |
| Resolution range | Autoscan 640x480 to 1920x1200, 480p, 576p, 720p, 1080i, 1080p |
| Return loss | -30 dB @ 5 MHz |
| DC offset (max. allowable) | 0.5 V |
| Video processing |  |
| Encoder (for scan converted output) | 10 bit digital |
| Digital sampling (for program and preview outputs) | 24 bit, 8 bits per color; 13.5 MHz standard (video), 194 MHz standard (RGB) |
| Colors (for program and preview outputs) | 16.78 million |
| Horizontal filtering (for scan converted output) | 3 levels |
| Vertical filtering (for scan converted output) | 3 levels |
| Encoder filtering (for scan converted output) | 3 levels |
| Video output |  |
| Number/signal type | 2 scaled RGBHV, RGBS, RGsB, HD YUV component video program outputs 1 scaled RGBHV, RGBS, RGsB, HD YUV component video preview output 1 optional DVI, HDSDI (SMPTE 292 M), or scan converted output (interlaced component video, S-video, composite video) |
| Connectors | (2) 15-pin HD (1 for preview output, 1 for program output) 5 BNC female (for program output) |
| Nominal level | 1 Vp-p for Y of component video and S-video, and for composite video 0.7 Vp-p for RGB and for R-Y and B-Y of component video 0.3 Vp-p for C of S-video |
| Minimum/maximum levels | 0.0 V to 0.7 Vp-p (RGB) 0.0 V to 1.0 Vp-p (component video, G of RGsB) |
| Impedance | 75 ohms |
| Scaled resolution | 640x4801,2,3,4,5,6, 800x6001,2,3,4,5,6, 852x4801,2,3,4,5, 1024x7681,2,3,4, 1024x8521,2,3,4,1024x10241,2,3, 1280x7681,2,3,4, 1280x10241,2,3, 1360x7651,2,3, 1365x7681,2,3, 1365x10241,2, 1366x7681,2,3, 1400x10501,2, 1600x12001,2, HDTV 480p2,10, 576p1,5, 720p1,2,8,9,10, 1080p1,2,7,8,9,10, and 1080i1,2,10 1864x10501,2, 1280x8001,2,3, 1440x9001,2,3, 1680x10501,2, 1080p Sharp2, and 1920x12002* 1 = at 50 Hz ; 2 = at 60 Hz; 3 = at 72 Hz ; 4 = 96 Hz; 5 = 100 Hz; 6 = 120 Hz; 7 = 24 Hz; 8 = 25 Hz; 9 = 30 Hz; 10 = 59.94 Hz |
|  | * 1920x1200 is available only on units shipped after first quarter of 2009. |
| Return loss | -30 dB @ 5 MHz |
| DC offset | ±25 mV with input at 0 offset (except for RGsB and component video) |
| Switching type | Seamless switching (cut or dissolve) |
| Sync |  |
| Input type | Autodetect (RGBHV, RGBS, RGsB), and component |
| Output type | RGBHV, RGBS, RGsB Tri-level on Y, R-Y, B-Y channels of component video or bi-level on Y channel of component video (selectable) |
| Standards | NTSC 3.58, NTSC 4.43, PAL, SECAM Optional SDI/HDSDI input: SMPTE 259M-C, SMPTE 292M |
| Input level | 2.75 V to 5.0 Vp-p for RGBHV or RGBS 0.6 Vp-p for component video tri-level sync 0.3 Vp-p for component video bi-level sync or RGsB |
| Output level | TTL: 5.0 V p-p, unterminated |
| Input impedance | 510 ohms, vertical; 75 ohms, horizontal |
| Output impedance | 75 ohms |
| Max. input voltage | 5.0 Vp-p |
| Polarity | Positive or negative (selectable) |
| Audio |  |
| Gain | Unbalanced output: -6 dB; balanced output: 0 dB |
| Frequency response | 20 Hz to 20 kHz, ±0.05 dB |
| THD + Noise | 0.05% @ 1 kHz at +4 dBu and 0 dB gain |
| S/N | >83 dB (typical) at maximum output (unweighted) |
| Crosstalk | <-80 dB @ 1 kHz, fully loaded |
| Stereo channel separation | >90 dB @ 1 kHz |
| CMRR | >75 dB @ 1 kHz |
| Audio input |  |
| Number/signal type | 6 stereo, balanced/unbalanced |
| Connectors | (6) 3.5 mm captive screw connectors, 5 pole |
| Impedance | >10k ohms unbalanced/balanced, DC coupled |
| Nominal level | +4 dBu (1.23 Vrms), -10 dBV (316 mVrms) |
| Maximum level | +19.5 dBu, (balanced or unbalanced) at 1% THD+N |
| Input gain adjustment | –18 dB to +24 dB |
| Audio delay compensation (automatic) | 20 ms when scaling RGB computer video or HDTV component video 75 ms when scaling NTSC component video, S-video, or composite video 90 ms when scaling PAL component video, S-video, or composite video |
| NOTE; 0 dBu = 0.775 Vrms, 0 dBV = 1 Vrms, 0 dBV ≈ 2 dBu. |
| Audio output |  |
| Number/signal type | 3 stereo, balanced/unbalanced (2 program, 1 preview) |
| Connectors | (3) 3.5 mm captive screw connector, 5 pole |
| Impedance | 50 ohms unbalanced, 100 ohms balanced |
| Gain error | ±0.5 dB channel to channel |
| Maximum level (Hi-Z) | >+21 dBu, balanced or unbalanced at 1% THD+N |
| Maximum level (600 ohm) | >+15 dBm, balanced or unbalanced at 1% THD+N |
| Control/remote — switcher |  |
| Serial control ports | 1 RS-232/RS-422: 1 rear panel 9-pin female D connector 1 RS-232: 1 front panel 2.5 mm mini stereo jack |
| Baud rate and protocol | 9600 (default), 19200, 38400, 115200 baud; 8 data bits; 1 stop bit; no parity |
| Serial control pin configurations |  |
| 9-pin female D connector | RS-232: 2 = Tx, 3 = Rx, 5 = GND RS-422: 2 = Tx-, 3 = Rx-, 5 = GND, 7 = Rx+, 8 = Tx+ |
| Mini stereo jack | RS-232: tip = Tx, ring = Rx, sleeve = GND |
| Ethernet control port | 1 RJ-45 female connector |
| Ethernet data rate | 10/100Base-T, half/full duplex with autodetect |
| Ethernet protocol | ARP, ICMP (ping), IP, TCP, UDP, DHCP, HTTP, SMTP, Telnet |
| Ethernet default settings | Link speed and duplex level = autodetected IP address = 192.168.254.254 Subnet mask = 255.255.0.0 Gateway = 0.0.0.0 DHCP = off |
| Web server | Up to 200 simultaneous sessions 16 MB nonvolatile user memory |
| Program control | Extron's control/configuration program for Windows? Extron's Simple Instruction Set (SIS?) Microsoft? Internet Explorer? ver. 6 or higher, Netscape? Navigator?, Telnet |
| General |  |
| Power | 100 VAC to 240 VAC, 50-60 Hz, 45 watts, internal |
| Temperature/humidity | Storage: -40 to +158 °F (-40 to +70 °C) / 10% to 90%, noncondensing Operating: +32 to +122 °F (0 to +50 °C) / 10% to 90%, noncondensing |
| Mounting |  |
| Rack mount | Yes, with included brackets |
| Enclosure type | Metal |
| Enclosure dimensions | 3.5" H x 17.5" W x 12.0" D (2U high, full rack wide) (8.9 cm H x 44.4 cm W x 30.5 cm D) (Depth excludes connectors and knobs. Width excludes rack ears.) |
| Product weight | 8.5 lbs (3.9 kg) |
| Shipping weight | 18 lbs (9 kg) |
| DIM weight |  |
| USA/Canada | 15 lbs (7 kg) |
| International | 18 lbs (8 kg) |
| Vibration | ISTA 1A in carton (International Safe Transit Association) |
| Regulatory compliance |  |
| Safety | CE, CUL, UL |
| EMI/EMC | CE, C-tick, FCC Class A, ICES, VCCI |
| MTBF | 30,000 hours |
| Warranty | 3 years parts and labor |